Outagamie County Carbon Monoxide Detector Guidance (TRH’s)

Where are Carbon Monoxide Detectors Required?

Definitions:

- “Carbon Monoxide Detector” means an electronic or battery-operated device that sounds an alarm when an unsafe level of carbon monoxide is in the air.

- “Fuel-Burning Appliance” means a device that is used or intended to be used in a residential building and burns fossil fuel or carbon based fuel where carbon monoxide is a combustion by-product. “Fuel-burning appliance” includes stoves, ovens, grills, clothes dryers, furnaces, boilers, water heaters, heaters, and fireplaces.

- “Unit” means a part of a residential building that is offered for pay as a sleeping place or sleeping accommodations to an individual or a group of individuals maintaining a common household, to the exclusion of others. It includes, but is not limited to, an individually rented room or suite of rooms in a hotel or an individually rented tourist cabin or cottage.

- “Sleeping Area” means the area of the unit in which the bedrooms or sleeping rooms are located. Bedrooms or sleeping areas separated by another area such as a kitchen are considered separate sleeping areas.

Carbon monoxide detectors are required in all Tourist Rooming Houses that have a fuel-burning appliance or an attached private garage. Depending on the layout of the house, multiple detectors may be required. A Carbon monoxide detector shall be installed in the following places:

- Outside each separate sleeping area in the vicinity of the sleeping rooms.

- In the sleeping rooms if there is a fuel-burning appliance within the room or its attached bathroom.

Carbon monoxide detectors are sensitive to cold temperatures and should not be allowed to freeze as cold temperatures may cause them to malfunction. Detectors also have an expiration date that is sometimes stamped on the back or in the user manual, after this time they are no longer reliable and may malfunction at any time, so it is important to check and replace them frequently.

Contact Outagamie County Public Health if you have any questions at (920) 832-5100.